MOUTH OF COLUMBIA RIVER BENEFICIAL USE PROGRAM

LOWER COLUMBIA SOLUTIONS GROUP WORKSHOP

January 14, 2020, 9:00 am – 3:00 pm

Holiday Inn Express; Astoria, Oregon

WORKSHOP SUMMARY

SYNOPSIS

The Lower Columbia Solutions Group (LCSG) conducted a January 14, 2020 workshop to explore the opportunities and constraints to federal funding for nearshore and on-shore beneficial use projects for the disposal of dredged materials from the Mouth of the Columbia River, as well as to develop plans for 2020/2021 beneficial use projects in the nearshore and on-shore along the Oregon and Washington coasts.   The workshop was organized around four key objectives:

- Assuming that we have progressed from a series of pilot projects to a routine disposal and monitoring program at the South Jetty Site, what is the threshold for no harm in terms of disposal volume?  Are we comfortable that the site’s recertification process scheduled for 2022 will provide the appropriate process and timing for assessing the need for adaptive management of disposal and monitoring at this site?

Response:

- Oregon’s water quality certification authorizing the project will be due for renewal in 2022; recommended changes should be identified now for consideration in the re-certification process.

- Based upon research and monitoring results to date, additional monitoring is not a priority at this site.  The group agreed that the project has transitioned from a pilot project to a routine part of the MCR beneficial use program.

- The current permit authorizes up to 500k cubic yards.   The consensus of the group two years ago was that, given the goal to minimize the material sent to the Deep Water Site,

increasing the maximum capacity to 600k cubic yards should be considered.  Discussion focused on the 600k cubic yards being an upper limit rather than an annual amount, with the expectation that in most years the volume would be less.  The 600k figure at the one foot height increase in mounding threshold appears to be a safe upper limit that avoids adverse impacts to the fishery and to navigation safety.

- Also proposed is a larger site area in order to place a greater amount of material without creating mounding impacts.

- It was agreed that the recertification process would be the appropriate time to evaluate any future increase in the site’s footprint and maximum volume.

- What can we conclude about the North Head Site from the two-year pilot project? What are the next steps for this site?

Response:

- The information collected through the pilot project will be useful in designing a well-trained model that could help define a final placement area and approach.

- The two-phase pilot project was intended to:

- Be an experiment to measure dispersion area, with the idea that some sediment might feed Benson Beach and the shoreline to the north.

- Provide a large enough study area to reflect the variable various ocean conditions.  A combination of factors led to the identification of the current study area.  The crab fishery pushed for an area north of North Head, while the technical team argued to be south of it.  The compromise was to make a big site on both sides.  Peacock Spit is very energetic and that is where the waves are breaking most wildly.  Operationally, a site for dredge material placement in the area it needs to be reliable.

- Assess the effects of low-relief accumulation on the seabed using thin-layer placement.

  - Assess whether there is a preferential transport direction based on mound: transect orientation, i.e. will sediment placed in a E-W (cross-shore) berm be dispersed more readily than material placed in a N-S (along-shore) berm.

  - Determine whether a hopper dredge can operationally place sediment N-S (along shore) at MCR.

  - Assess whether the northern end of the NHS exhibits a similar sediment transport rate as the southern end.

- Pilot project findings include:

- It’s a dispersive site, with sediment placed on both E-W and N-S lines dissipating at the same rate.  It appears that the NHS s less dispersive than the Shallow Water Site, however.

- A placement volume of 400-500 KCY/year appears to be below a threshold of concern, with no demonstrated effect on wave amplification based on 2 feet of mounding of dredged material in 35-50-foot water depth.

- It does not appear that sediment is being more preferentially transported with different placements.

  - Given good weather conditions, a hopper dredge can operationally place sediment N-S (along shore) at MCR.

- Placement at the southern end of the NHS would appear to have the greatest potential benefit to Benson Beach.

- Use of the North Head Site will not significantly reduce erosion at Benson Beach.  If we want to specifically reduce erosion at Benson Beach, we need to not only place more sediment but place it more efficiently.

- For future placements in the area, it was suggested:

- Use sub-areas of the North Head Site on a rotational (annual) basis at 300-500KCY/year, with close stakeholder coordination.  This would minimize impacts to each sub-area and would be a good compromise to avoid impacts to fisheries.

- Place materials in other than a straight line.

- Place different volumes of material within different portions of the NHS, eg 600kCY/year in the southern third of the North Head Site and less in the northern third.

- Evaluate the potential risks of increasing the volume of material placed above 400-500 KCY/year.

- Assess the potential to expand the study area to the south closer to the North Jetty, recognizing the presence of various shipwrecks.  The mobility of sediment is much higher in the area to the south.  Sediment would be expected to leave the site at a much faster rate so there would be fewer impacts, including minimal impacts to crabs/crabbing.

- Permit issues associated with expansion of the study area include:

- Evaluating what was authorized in the pilot project certification.  The Cert is intended to define a permanent site.

- To expand the pilot area would require either amending the Cert or addressing a larger study area when it is reissued in 2023.  If an amendment of the Cert in the short term is desired, Ecology would need to receive a proposal / report generated from the research and ongoing discussions.

- There is flexibility written in the Cert for greater volumes within the current study area with guidance from LCSG.  One of the conditions requires the Corps to work with LCSG to determine the site area and volumes.

- Amending the Cert to enable a Phase III project intended to provide more information on how to use the site, eg three sections for rotational placement.

- It was agreed to convene the technical team to advise on design of a placement program at the currently authorized North Head pilot project site in 2020 (Phase 3) that considers:

- That we're learning something new about the site and the dispersal of material placed there.

- Considers increasing the volume placed within the existing designated site, e.g., placement of 2' of material over a wider area.

- Considers a longer-wider feature along the southern border of the study area.

- Assesses the viability of expanding the current placement area in the future, specifically to the south.

- If a Phase 3 pilot project is proposed, Ecology will require a proposal, monitoring report and description of desired outcomes.

- Based on what have we learned today about RSM authorities and funding opportunities, what are the strategies and next steps for establishing a sustainable beneficial use program at MCR?

Response:

- WDOE and the Corps are developing a plan to be ready with an application if another WRDA Section 1135 or other funding method becomes available.  At the same time, the Corps will continue to include a pump ashore project in the budget in hopes that funds do become available.

- Annual contracts are challenging and time consuming.  There is a desire to move more toward a package of services that can be outlined in the RSM plan to authorize the work already being done.  A three- to four-year plan with objectives to reach in that time period would be much more efficient and cost-effective.  Because the Corps budgeting is on an annual basis, funding a five-year program would be challenging.  However, a five-year program could be contracted / budgeted for annually.

- Proposals to the RSM program are due in July and small-scale research monitoring programs need to be under $150k.  Participants were encouraged to think about any options for potential projects for RSM funding.

- The national RSM program funding could be a source to help fund producing brochures, slideshows, etc.  The Portland District/LCSG should put in a proposal for outreach materials, as well as for staffing support for this collaborative effort.

- Investigating similar projects around the country to see how they are funded was suggested.

      - Regular communication from Corps staff is critical to staying on top of the budget process / deadlines.

- Participants agreed that staying connected to the national RSM program could benefit and supplement the MCR work.

- What is the overall strategy and the next steps for implementing onshore placement at Benson Beach?

Response:

- Construction of the North Jetty has changed the Peacock Spit, with waves / currents beating it down and diffusing it out.  As the morphology changes, the patterns affecting Benson Beach change.   Despite the placement of approximately 1 Mm3 of sediment in the Shallow Water Site annually between 2014 and 2019, Benson Beach continued to erode 420,000 m3/yr.

- Connectivity between current dredge placement areas (Shallow Water and North Head sites) and Benson Beach is relatively low.  Placing dredge material between the North Head and Shallow Water Sites would most efficiently enhance the sediment budget of Benson Beach.

- Erosion at Benson Beach is so significant that the need for on-shore placement can be justified based on safety and economic concerns, including loss of camping/other park infrastructure, loss of state park revenues, safety to the public, etc.

- The prior Benson Beach onshore placement permit has expired and would need to be redone.

- The Portland District includes the Benson Beach onshore placement project in its budget request every year but is not funded because it’s not tied to navigation needs.  The squeaky wheel approach and unified messaging are needed to successfully compete for funding.

- It was suggested that Benson Beach placement needs to be viewed as helping the North Jetty.  However, tying Benson Beach placement to the jetty may not be the best approach because the Corps is just wrapping up jetty work.

- Tangible next steps include Corps/LCSG coordination on a funding request, development of unified messaging, and advocacy with the Congressional delegation and other elected officials.  It was suggested that the LCSG write a letter(s) to Corps leadership outlining the concern about Benson Beach.  Part of messaging/advocacy should be identifying/quantifying the potential losses and impacts of continuing erosions.  For the next budget cycle, this messaging is critical within the next couple of months.

- Corps staff indicated that the Portland District’s 2022 budget request has already been submitted, but that it could probably still submit a work package for 2022.  A plan would be needed asap. RSM program staff could be advised that something may be coming in and to hold off on submitting the final work package.  Corps staff will take the lead on this and will look at the justification from the prior onshore placement for background.

- What are the next steps for this group? Timing and assignments?

Response:

- Updating the RSMP to include the current science was identified as critical justification for MCR funding.  Jim will be query members about what elements to be updated and how, will circulate a draft, and then present an updated RSMP to the LCSG for approval.

- The Portland District is intending to assume the funding/contracting for facilitation services.  It is important that a new facilitator be aboard by June so that the program does not lose momentum.

- The technical team will be convened within the next couple of months in at least in a conference call to design the next steps for North Head Site placement and to help identify needed updates to the current RSMP.  It is anticipated that the full LCSG will probably meet again in June and December.

ATTENDEES

Washington Department of Ecology:  Brian Lynn, Bobbak Talebi, George Kaminsky, Penny Kelly, Tressa Arbow, Sonni Tadlock

Washington Department of Fish & Wildlife: Lauren Bavernschmidt

Washington State Parks & Recreation:  Evan Roberts, Josh Bell, Matt Niles

Washington Sea Grant:  Jackson Blalock

Oregon Department of Land Conservation & Development:  Patty Snow, Nick Tealer, Andy Lanier, Lisa Phipps

Oregon Sea Grant:  Patrick Corcoran

Oregon Regional Solutions:  Jennifer Purcell

Corps of Engineers:  Jarod Norton, James McMillan, Rod Moritz, Rachael Stolt, Terry Geroux, Karla Ellis, Andrew Bran, Kath Groth

Environmental Protection Agency:  Bridgette Lohrman

NOAA Fisheries:  Curtis Roegner

Columbia River Crab Fishermen’s Association:  Dale Beasley

Columbia River Bar Pilots: Steve Ackerman

CREST:  Denise Lofman, Justin Saydell

Surfrider Foundation:  Gus Gates

Anchor QEA:  Kim Marcotte

Solmar Hydro:  Jason Magalen

Senator Murray:  Bryan Stebbins

Facilitation Team:  Jim Owens, Alisha Morton

WELCOME, INTRODUCTIONS AND MEETING PURPOSE

Patty Snow, Oregon Coastal Program Manager and co-convener, welcomed participants and asked for a round of Introductions.

Brian Lynn, WDOE and Co-convener, then provided a brief history of the Lower Columbia Solutions Group (LCSG).

- The effort started in 2002 as a bi-state effort that was convened by the two governors at the time.

- Purpose was to focus on challenges in the Lower Columbia River, i.e. channel deepening.

- Funding has never been consistent nor firmly established and has varied over the years.

- LCSG consists of state, local and federal entities and other non-governmental groups.  It hasn’t always been easy. Some relationships were tense / not positive, and this effort helped overcome that.

- After channel deepening, LCSG really focused on regional sediment management and ultimately adopted a Regional Sediment Management Plan (RSMP) for the mouth of the Columbia River in 2011.

- We are in the midst of implementing the RSMP.  We have all this clean, beautiful sand that comes out of the Columbia and we’re determining what to do with that sand, how to use it in the most beneficial way possible.  Sustaining beaches, reducing erosion and shoring up the jetties are the best uses.  We are trying to achieve those beneficial uses in the best possible ways.

- At the North Head Site, a pilot project is underway.  We’ll be discussing that site in this meeting.  At Benson Beach, we have significant, ongoing beach erosion but challenges in terms of cost to place sand ashore and we want to continue the conversation today about what we do there.

- RSMP identifies current and future potential disposal sites to keep the sand in the system.  It is built around an adaptive management process.  In 2015, we came back to this group to confirm if we are all in this and whether this work is still important.  There was a resounding nod from the group that it’s really helpful to have a third party that is more neutral coordinating the effort. Jim’s role has been key in this process.

- The two Governors offices are no longer engaged like they were historically and the convening is now provided by the two states’ coastal management programs.

- This past year we attempted to put in place cooperative agreements with agencies within the two states and it turned out to be a logistical nightmare.

Patty reviewed the meeting objectives:

- Assuming that we have progressed from a series of pilot projects to a routine disposal and monitoring program at the South Jetty Site, what is the threshold for no harm in terms of disposal volume?  Are we comfortable that the site’s recertification process scheduled for 2022 will provide the appropriate process and timing for assessing the need for adaptive management of disposal and monitoring at this site?

- What can we conclude about the North Head Site from the two-year pilot project? What are the next steps for this site?

- Based on what have we learned today about RSM authorities and funding opportunities, what are the strategies and next steps for establishing a sustainable beneficial use program at MCR?

- What is the overall strategy and the next steps for implementing onshore placement at Benson Beach?

- What are the next steps for this group? Timing and assignments?

Patty and Brian reminded the group that this is Jim’s last meeting with us. Dale Beasley said he’d like to personally thank Jim Owens for his many years of service to the group.

Jim said it’s fair to say that Jarod Norton’s role as the MCR project manager helped moved all of this along exponentially from where it had been and helped us build a much closer and healthier working relationship with the Corp.

PRESENTATION AND FACILITATED Q/A

Overview of National RSM program

Jarod gave an overview of the national RSM program.  He said staff from the Corps national RSM program in Washington, DC was intending to attend but there was a medical emergency that prevented them from coming.  The full PowerPoint Presentation can be viewed here: https://lowercolumbiasolutionsdotorg1.files.wordpress.com/2020/05/1-national-rsm_mcr-meeting.pptx

Discussion

- Bridgette Lohrman, EPA, asked if funding for RSM projects is one time or ongoing. Jarod said they are one-time projects. The District puts in annual proposals for RSM program funding; it is not part of the O&M budget cycle.  We’ve had favorable contracts for dredging bids and can fund our own RSM projects. It’s been really beneficial to the region to have flexibility in the use of the excess funds.

- Brian asked if RSM funds can be used to fund stakeholder groups like this one. Jarod said yes, the national RSM likes these collaborative efforts.

- Bobbak Talebi asked if the national RSM program likes to see annual projects?  Jarod said our group is somewhat of a poster child for this, as we routinely meet together to talk about how we do better as a region. We are representing the West Coast in what we’re doing. They are going to use our work as a case study. We are seeing benefits, reductions in cost but we still have to ask for special funds, not just operation funds.

- Bobbak said Grey’s Harbor was awarded a Section 1122 project.  WDOE is leading the environmental permitting as there are funds for that.  But the funding for design and construction has not come through yet.  Jarod said there is only about $6 million for the projects that were awarded, when $80 million was expected.

- Brian said they are hearing it is unlikely there will be another round of Section 1122 funding.   Jarod said he hasn’t heard that yet.

- Jarod said that RSM projects don’t have to be something that is currently being done. He said if anyone thinks there is a project or idea that should be funded by the RSM program, they should reach out to Katherine in the national RSM office.  He said she is open to changing the structure of how things have been done in the past.

NWP RSM Program Opportunities and Constraints

Rod Moritz gave a presentation on the NWP RSM Highlights.  The full PowerPoint Presentation can be viewed here: https://lowercolumbiasolutionsdotorg1.files.wordpress.com/2020/05/2-norton-nwp-rsm-work.pptx

Rod said the term RSM was coined in this area and Benson Beach got its start with RSM as a national project.  He said we got started with 40k cubic yard placement in 2002 at Benson Beach. Jim added that in 2007 we placed 35k cubic yards at the South Jetty Site as a pilot project.  This was followed up with a series of Science Policy Workshops, which brought folks in nationally.  That lead to a second South Jetty Site project in 2012, again at 35k cubic yards to see if there was beneficial use.

Discussion

- Brian said the recent Clatsop Spit project seems like it’s solving an important concern, but is it a maintenance issue tied to the jetty?  And what’s the difference between Clatsop Spit and Benson Beach? Jarod said the Clatsop Spit project is definitely tied to the jetty.  Benson Beach is looked at differently even though it would help the jetty.

- Brian asked if there are conversations we should be having about changing the view of Benson Beach placement to help the jetty.  Jarod said we could change the justification to being tied to the jetty or we could keep the justification the same but get elected officials and local groups pushing for its funding.  He said the erosion at Benson Beach is so bad now that you can justify the need for based on safety concerns, losing campsites, etc. Jarod said we just have to be a squeaky enough wheel. He said we could change the approach by tying it to the jetty but it’s not a great approach because the Corps is just wrapping up the jetty work.  Jarod said unified messaging is needed.

- Patty asked if more cobble is being placed at Clatsop Spit this summer.  Jarod said yes as part of the current jetty work project.  He said the layers are replicate of natural conditions.  He said a week after the cobble is placed, you have no idea there was a construction project.  He said it’s really impressive.

- Jarod said sand fences are wooden structures held together by wire that are really beneficial.  They work great on the east coast, but for some reason people don’t like them here.  They don’t cost a lot and they save money on dredging / dispersal costs.  Karla Ellis said the biggest challenges with the FY14 Oregon Shoreface Stabilization project was working with the local entities.  She said with one of the challenges being that Port managers have turned over very rapidly, carrying a consistent message is hard to do.  Jarod agreed and said it was a slam dunk project that hit many roadblocks.  He said it could be a big benefit.

- Jarod said he wanted to share the scope of what USACE has done with RSM program since 2012 to hopefully trigger something that this group would like to do.  He said it can be MCR-specific or anything else participants think should be happening.

- Brian said we will have conversation about the range of things, and this will be one focus area.  He asked participants to think about any options for potential projects for RSM funding.

- Brian said we have surplus O&M funds now and asked when do we decide when a project warrants a RSM proposal.  Jarod said it is not a super clear answer.  If we have a scope and an estimated cost, we can flesh that out.  O&M funds will be tied back to dredging so we can justify it there.  He said there is a review process when the Corps moves money around, so the justification has to be tied back to the original project.

- Brian asked if there is a way for this group to weigh in on proposal submittals to the national RSM process.  Jarod said there isn’t currently, but we would have to look to Katy for that.  He said they have already looked at 65 proposals this last year.

- Bobbak said if there are any projects around the country that we could look at like ours and see how they are funded.  Jarod said there are some ongoing projects but those are O&M funded.  He said in cases like ours and the South Jetty Site specifically, the funding gets tied back into O&M because it is tied to dredging and channel maintenance.  Bobbak asked if we could tie funding to the RSMP.  Jarod said RSM funds were used when it was exploratory but once we have a permit for dredging and disposal, it’s no longer a research project.  Rod said the science we’re doing is important and it’s essential to take actual science into the policy / permit process.  He said we have to have the justification to even go after the permit.  Bobbak said updating the RSMP seems pretty essential to get the current science in there as justification for funding.

- Brian said we want to figure out how to continue to help our Corps partners tap into the program and get what we need to advance this work.  He said it was helpful to see what we have already done and to consider if there is an ongoing way for partners to engage in the process and contribute to applications submitted.

PANEL DISCUSSION

Results of 2019 Field Season / What has been learned from South Jetty Site and North Head Site Pilot Projects

Overall Program Results

Jarod gave a presentation focusing on overall program results for 2019.  The full PowerPoint Presentation (Norton – NHS LCSG.PPTX) can be viewed here: https://lowercolumbiasolutionsdotorg1.files.wordpress.com/2020/05/3-norton-nhs-lcsg.pptx

Jarod summarized the projects completed at the South Jetty Site and the North Head Site.  He said the plan at the South Jetty Site was to place 400k cubic yards with thin layer placement to maximize the quantity that was going in there.  He said the permit is capped at 500k cubic yards.  Due to issues with the Essayons, the placement time was shortened but were able to place 280k cubic yards.  We want to spread material around this big site to minimize mounding and impacts to crabs.  The water quality cert will be due for renewal in 2020 so if any changes are recommended that would be the time to make them, rather than recertifying in 2023.

For the North Head Site, Jarod said they placed 50k cubic yards parallel to the shore and an additional 50k cubic yards perpendicular to the shore to evaluate if the orientation of the disposal has any impact on dispersal.  He said the dredge got out there late and the weather was not ideal.  Jarod said this was a good test even in good weather as they were running really precise disposal lines.

Jarod said they were able to maximize placement at South Jetty and North Head sites as much as possible.  He said they only took five loads offshore to the Deep Water Site this year.

Solmar Hydro Survey Work

Jason McMillan gave a presentation on the survey work that Solmar Hydro under contract with the Corps.  The full PowerPoint Presentation (SHI_NHS_20200114.pptx) can be viewed here: https://lowercolumbiasolutionsdotorg1.files.wordpress.com/2020/05/4-shi_nhs_20200114.pptx

Jason said to avoid equipment loss, they proposed putting out a wave buoy that has ADCP to measure currents magnitude and direction. Solmar Hydro has a data collection tripod contract with the Corps..  From August 28 – October 1, they measured 4 things: Surface Directional Wave Spectra, Water Column Currents, Fixed-depth CTDs and Water Column CTD.  He said they worked with Job Corps Ironwood to place the buoy as a training experience for students.

Jason said the big take home is that, during this time period, the dominant current direction was towards the southeast.  He then gave a summary of the what they learned:

- Wave heights generally 1-3 meters

  - Increased in height mid-deployment (9/15/2019)

  - Large Hsig/Tp event ~9/4/2019 (Hs up to 3 m, TP up to 20 sec)

  - Agreement with CDIP buoy

- Currents

  - Fluctuate E-W with tides

  - Predominately directed toward south

  - Mean Ve = 4.5 cm/s (east)

  - Mean Vn = 15.3 cm (south)

  - Mean Vu = 6.2 cm/s (up)

  - Mean Vavg = 22.9 cm/s (SSE)

- CTD

  - Temp near-surface and mid-column 14-17ºC throughout

  - Near-bottom temperature increased mid-month to near agreement w/ surface

  - Salinity near-surface fluctuated 20-30 psu

  - Salinity mid-column remained relatively high (27-31 psu)

  - Salinity near-bottom high near beginning deployment (27-30 psu) and dropped to ~27 psu after 9/15

Discussion

- Rod said the currents are really interesting moving to the southeast at that location.  CTD temperature profile shows a very sustained dynamic change.  It’s a science picture that could help us understand what is happening at Peacock Spit ecologically.

- Rod said we only left the buoy out for a short period of time because we lost a buoy before and we want to avoid that.  He said the first step is getting the data.  If they could replicate that month then we could use the model to look at times when we don’t have data.

- Jason said the data is very similar between that from their buoy and the CDF buoy off the mouth of the North Head Site so that helps with using this data for modeling.

- Dale said the data set does not really capture large winter storms.  He said he has never chased drifting crab pots to the south but instead it’s always to the north.  He said there is more sediment movement in one of these large storm events then you will get all year.

- Curtis said the currents are part of the broad circulation patterns.  He said most of the energy that will move things comes in from the south with winter storms.

- Patty said the CDF buoy is out there year-round.  She asked if we could look at the winter data from the CDF?  Jason said it will give you limited data but not for the water column.

- Bridgette said the assumption is the sediment is moving south with this experiment.  She asked if that is what we expected to see or was it different than we thought.  Rod said we expected to see southern current at that time of year.  He said the waves are coming in from the north / northwest.  He said there are environmental shifts at the end of October to November and it becomes progressively northward.  He said we have more information that we had before and some of this could be used to make a well-trained model for wave and circulation.  Bridgette said it could help define the final placement study.

MCR Crab Telemetry Study

Curtis Roegner, NOAA Fisheries, gave a presentation on the survey work that has completed on the MCR Crab Telemetry Study.  The full PowerPoint Presentation (Roegner MCR tech meeting 2020.pptx) can be viewed here: https://lowercolumbiasolutionsdotorg1.files.wordpress.com/2020/05/6-roegner-mcr-tech-meeting-2020.pptx

Curtis said they have been working on this project in various aspects since 2014 and are now moving into acoustic tags in order to look at migration patterns.  The various research studies demonstrate that thin layer deposition is relatively benign to the crabs.  The wall of sediment impacts them, but they return to the site within 30 minutes to one hour.  He said there isn’t really a negative effect.

Curtis then gave an overview of the project history.

- In 2015-2016, at the South Jetty Site they did tagging at the impact and control sites.  There was not much difference between impact and control.  Crabs are pretty active and moving really fast out of the arrays.

- In 2018, as part of the North Head Site mound experiment, arrays were placed around the dredge track.  Previous depositions were a one-shot deal but here the dredge was going back over the same place again and again.  We really wanted to see if there were any accumulative effects on crabs.  They set up other receivers nearshore in three spots to see where the crabs were moving outside the area.  They moved out of the array quickly when they were getting dumped on.  At the two controls, they did not move out as quickly and meandered around the array more.

- The 2019 mound experiment at both the South Jetty and North Head sites show that crabs are being impacted when sedimentation is going on.  They move out quicker and are going north.  Crabs are not going with the current but rather migrating north but the currents are going south.

Curtis then summarized the findings to date, along with opportunities for 2020

- Summary of tag studies:

  - Acute impact variable

  - No evidence of increased mortality

  - Crabs highly mobile

  - Northward migration

  - Limited import / export from estuary

- Mound experiments (preliminary)

  - Similar to residence in array between control and impact

  - Impact – strong northward migration

- Unknowns

  - Extent of northward movement

  - Cumulative effects

- Opportunity for 2020

  - Examine large-scale migration patterns

  - Fill in gap between MCR and Olympic Grid

  - Crab behavior and low do patterns

Discussion

- George Kaminsky asked if the buoys in the Olympics are staying out.  Curtis said yes and that they bring them in in March.

- Gus Gates asked which buoys have been picking up the great white sharks.  Curtis said they see them year after year at the South Jetty Site.  They also see them at Cape Falcon.

- George said Jason’s data shows southward currents, but the movement of crabs is to the north.  Curtis said he thinks they are hungry and searching for food.  He said if you go into the current, you’re getting easier access to food, as it’s moving towards you in the current.

North Head Site Results

Rod Moritz, Corps, gave a presentation on the work completed for phase II at the North Head Site (NHS).  The full PowerPoint Presentation (North_Head_Site_phase_II.pptx) can be viewed here: https://lowercolumbiasolutionsdotorg1.files.wordpress.com/2020/05/7-north_head_site_phase_ii.pptx

Rod said the North Head site was chosen so we would have a large enough study area to do the following things, with the idea that some sediment might feed Benson Beach and also north.  The initial concept for the nearshore placement at the North Head Site:

- A one-time experiment to measure dispersion

- Low-relief accumulation on seabed using thin-layer placement

- No effect on wave amplification:  2 feet of mounding of dredged material in 35-50-foot water depth will not affect wave action.

Rod said phase II was constructed based upon motivating questions from phase I:

- Is there a preferential transport direction based on mound: transect orientation?

  - Will sediment placed in a E-W (cross shore) berm be dispersed more readily than material placed in a N-S (alongshore) berm?

- Can a hopper dredge operationally place sediment N-S (alongshore) at MCR?

- Does the northern end of the NHS exhibit a similar sediment transport rate as the southern end?

Rod said five days after placement, the sediment can be seen moving from the transect shoreward.  He said wave action equates to sediment movement and the waves stir the sediment.  It’s dispersive.  Rod said both E-W and N-S lines are dissipating at the same rate.

Rod said we now need to consider where to go from here.  We are done with phase II.  He suggested using sub-areas of the North Head Site on a rotational (annual) basis at 300-500KCY/year with close stakeholder coordination.  This would minimize impacts to each sub-area and would be a good compromise to avoid impacts to fisheries and fisherman.  He said placement at the southern end would benefit Benson Beach the most.

Discussion

- Brian asked if there is modeling work being done on the sediment movement.  George indicated that he is going to talk about that.

- Rod said the deepest part of the outer parts are near 60 feet.  Patty asked where the depth of closure is in the study area.  Rod said it’s the 58-60 foot contour and that is where a lot of sediment change occurs.

- Jason asked why we think the sediment is being lost from Benson Beach in the first place and if this will be a bandaid or will it help in the long run if we place sediment in the southern end of the North Head Site.  Rod said some of it moves south to the channel, but a lot is being pulled off and moved to the north.  He said during bigger storm events it is being pulled north.  He said different sediment moves differently.  Rod said over time (decades), it is moving more to the north than anywhere else.

- Curtis asked if Peacock Spit is gone because of lack of sediment and not being able to replenish it.  Rod said that Peacock Spit did not exist this way prior to the construction of the North Jetty.  He said Peacock Spit is changing.  It was an unnatural morphology and now the waves / currents are beating it down and diffusing it out.  As the morphology changes, the patterns affecting Benson Beach change.

- Brian said the underlying approach going forward would still be thin layer placement.  He said one purpose of the pilots was to see where the sand was going.  Rod said if we were to go forward, we may want to do other things besides placing in a straight line.  He said it does not appear that sediment is being more preferentially transported with different placements and we did not know that before this pilot study.

- Jim asked if we would maintain the two-foot boom height.  Rod said we would emulate what we do at the South Jetty Site.  He said we could split it into cells and make sure it’s thin layer placement.  He said it does the least amount of harm and we certainly don’t want to create any mounding.

- Jim asked if the southern third of the North Head Site could perhaps take 600 KCY/year and maybe less in the northern third.  Rod said that is a discussion for the group at large.  He said it seems like 400-500 KCY/year is below a threshold of concern but to go higher than that the group needs to evaluate any potential risks.

LUNCH

Results of collaborative beach and nearshore monitoring over the past six years with North Head sediment transport modeling

George Kaminsky gave a presentation on the Observations of Coastal Change and Numerical Modeling of Sediment Transport Pathways at the MCR.  The full PowerPoint Presentation (MCR_LCSG_2020_01_14_Kaminsky.pptx) can be viewed here:

https://lowercolumbiasolutionsdotorg1.files.wordpress.com/2020/05/8-mcr_lcsg_2020-01-14_kaminsky.pptx

George said this modeling work cooperatively done by USGS, Ecology, Deltares and OSU. Surveys are performed annually between July and September as part of a broader survey of beaches throughout the CRLC.  There have been six surveys between 2014 and 2019.

George summarized the beach and nearshore monitoring:

- Despite the placement of over 1 Mm 3 of sediment in the Shallow Water Site annually between 2014 and 2019, Benson Beach continues to erode.

- Additional sediment supplies and efficient transport between nearshore placement areas and Benson Beach are needed to overcome a 420,000 m3/yr deficit.

- Numerical modeling is used to evaluate and identify nearshore dredge placement areas that efficiently enhance the sediment budget of Benson Beach.

He then summarized the results of the Sediment Transport Pathways Analysis:

- Total of 19,000 source pathways were computed (19 wave classes, 2 discharge scenarios, and 500 sources per simulation).

- Sources originating within the North Head Site were primarily directed northward

- Sources that interacted with Benson Beach originated within the inlet, Shallow Water Site, along Peacock Spit, and from coastal areas to the north including the NH Site.

- Sources that interacted with the navigation channel originated within the estuary, MCR, and ebb delta.

George offered the following conclusions:

- Despite the placement of approximately 1 Mm3 of sediment in the Shallow Water Site annually between 2014 and 2019, Benson Beach continued to erode 420,000 m3/yr.

- Sediment transport within the NH Site is driven by wave events and fluxes are low suggesting the NH Site is less dispersive than the Shallow Water Site.

- Connectivity between current dredge placement areas (Shallow Water and North Head sites) and Benson Beach is relatively low.  Placing dredge material between the North Head and Shallow Water Sites would most efficiently enhance the sediment budget of Benson Beach.

He then discussed proposed next steps:

- Continue to monitor coastal morphology at Benson Beach and South Jetty in 2020.

- Examine conditions and predict fate of 2019 pilot placement.

- Compare model predictions of waves and currents to field data collected by the Corps at the North Head Site.

- More detailed point source analysis and comparison of sediment mobility to Shallow Water Site placement management.

Discussion

- Brian asked if there were other reasons why we didn’t define a placement area closer to the jetty.  Dale said there is a shipwreck right in the middle of the box that George drew in between the two sites.  He said they have done surveys and know where the wreck is now.  Jarod said he thinks there is also a shipwreck at the southeast corner of the North Head Site that we have to avoid.  Dale also said it’s a high fishing area in between the sites but not different than the North Head Site.

- George said the mobility of sediment is much higher in the area he’s proposing (new box).  Sediment would leave the site at a much faster rate so there would be fewer impacts.  He said the size of the site and its dispersiveness should result in minimal impacts to crabbing.  Dale said the dispersion rate isn’t constant from year to year.  He said at the Shallow Water Site it has varied a couple million cubic yards per year in the past.  He said mounding on the north side of the Shallow Water Site is more significant than on the south side. They have to put a lot more sediment by utilizing the south side.  The further north you move, the more capacity you lose due to mounding.

- Curtis said the differences in dispersion are quite significant.  He said this seems like something we want to get right and is crucial for keeping the sand on the beach.

- George asked if the question is how much we want to do something good in the system or do we specifically want to protect Benson Beach and its infrastructure.  He asked if there is a specific purpose for feeding this section.  The present practice does not do that.  He said the use of the North Head Site will not significantly reduce erosion at Benson Beach.  If we want to specifically reduce erosion at Benson Beach, we need to not only place more sediment but place it more efficiently.

- Jason Magelon, Solmar Hydro, said onshore placement seems the way to go.  George said it is a cost issue.  He said annual nearshore placement is better than wasting it offshore but not as efficient as onshore placement.

Status of North Head Site Permitting

Penny Kelly discussed the 401 Certification (Cert) which is the document that authorizes the NHS pilot study. The pilot study has now been concluded. If the Corps desires to dispose within a larger study area, we would have to look at what was authorized in the Cert and work with the Corps to see if they want to move to a designated site to be used on a regular basis.  The Cert is intended to define a permanent site.

She said the size of the area can be changed, but we would have to go through some steps to amend the Cert or address it in the next round.  The current Cert was issued in February 2018 and expires in five years from that date.  In the next go around, it would be best to apply for the long-term program at the NHS.  If something is desired sooner, we would have to prepare a proposal / report generated from the research and ongoing discussions.

Discussion

- Brian asked if there is flexibility written into the Cert for greater volumes with guidance from LCSG?  Penny said there is flexibility to adjust the volume.  One of the conditions requires the Corps to work with this group to determine the site area and volumes. She said changing the pilot area to a designated use site is part of that overall structure.

- Jim asked if we went with George’s proposed site if it would have to be a new Cert.  Penney said most likely yes with a request or proposal from the Corps.

- Bobbak asked how soon before the next dredging season do we need to get you info.  Penney said soon. There is the support for the Cert to use the site as proposed based on the work that’s been done.  She could see if we can amend it or if it would require a new Cert.  A new one would take a couple months to prepare along with a 30-day public comment period.

- Jarod said this is a good topic for the next agenda item.  We need to consider what we want to do with the remaining three years on the Cert.  He said both sites are coming up for review and what changes do we need to update as part of the Cert.

GROUP DISCUSSION

MCR Moving Forward

South Jetty Site

- Brian asked the group if we have reached a point where work at the South Jetty Site is just part of our routine program.  Participants agreed.

- Brian said the permit allows for 500k cubic yards and we placed 285k in 2019.  This will need to be re-certified in 2022.  Between now and then, should that 500k threshold be even higher.  The consensus two years ago that we were ok with moving to 600k cubic yards, which will need to be reflected in the next Cert on 2022.

- Brian said that Karen Braby with ODFW emailed yesterday that additional monitoring is not a priority at the South Jetty Site and the recertification process would be a good time to look at the site adaptively to see if we’re comfortable with the footprint and volume.

- Bridgette said she did not remember this 600k cubic yards number and would like more information on it.  Rod said we wanted to set an upper limit.  He said there may be a year that we place a lot less and maybe we want to replace more the next year.  We would most likely not place 600k every year.  It’s within the annual use plan the Corps puts out every year.  He said they can change the management within a given site to prevent thresholds being exceeded.  Given the goal to minimize the material sent to the Deep-Water Site, it would be prudent to have a higher limit.

- Bridgette said it seems a little premature since we have never placed 500k.  Jim said the scientists said if we put more than 600k we would probably end up with mounding.

- Penney said we like at least 2-3 months lead time before the re-Cert.  Jarod said they generally like to start this process a year in advance.  Penney said it’s more how quickly the information that is already there can be pulled together.

- Dale said his own goal is to not have any impacts to the fishery and no significant mounding.  He said 600k is a safe upper limit at the one foot per year level.

- Rod said the provision with 600k would be that not do it every year and do not exceed any threshold, i.e. one foot height increase.

- Dale said Jarod sent him some bathymetry from the fall and he compared it to the baseline.  He said the management of that site so far is within the one foot height.  You may have some leeway but not sure how much.

- Brian suggested that we have a little more discussion.  This group will be meeting again before the process needs to start for the re-Cert.  We will table this as a proposal to consider as we go through the re-Cert process.

RSM Discussion

- Brian said staying connected to the national RSM program could benefit and supplement our work here.

- Jarod said WDOE and the Corps are trying to come up with a plan to be ready with an application if another Section 1135 or other funding method comes available.  At the same time, the Corps will continue to include a pump ashore project in the budget in hopes that funds do become available.

- Gus asked what the timing on placement onshore would be and if there would be closures to the public access.  Jarod said there would be closures due to heavy equipment.

- Dale asked what the status of the Benson Beach onshore placement permit is and Jarod said it would need to be redone.

- Jim said stakeholder outreach and education is a requirement.  He said we have never had the resources to do public education and prepare informational materials. He said the national RSM program funding could be a source to help fund producing brochures, slideshows, etc.  Jarod said they could definitely put in a proposal for outreach materials.

- Dale said convincing the public about onshore placement at Benson Beach shouldn’t be too hard.  If we can’t protect Benson Beach from continuing erosion, then the public won’t have ocean front camping.

- Bobbak said annual contracts are challenging and time consuming.  He would like to see us move more towards a package of services that can be outlined in the RSM plan to authorize the work Curtis and George are already doing.  He said George has been doing this monitoring since July, but we don’t yet have a contract for him.  He’d like to see a strategy laid out to avoid working with no contract.  He said may\be a five-year work plan could be set up.  Bobbak said Jarod has been super creative to get this work funded and he imagines it takes a lot of time on his end.

- Curtis agreed with this.  He said it would be a lot more cost effective if we could have a three- to four-year plan with objectives to reach in that time period.  Jarod said the Corps budgeting is every year so they can’t fund a five-year program, but they could develop a five-year program and then the Corps could work on contracting / budgeting yearly.

- Jarod said proposals are due for the RSM program in July and these small-scale research monitoring programs need to be under $150k.

- Brian said communication from Jarod / Katie along the way is really helpful to stay on top of the process / deadlines.

North Head Site Strategy Going Forward

- Brian said we need to keep using safe practices to feel good about more sediment disposal.

- Rod said that we need to be objective and see where the sand goes.  After seeing George’s results, why didn’t we look at that area in the first place.  Rod said we did but there was a combination of things that pushed us to the north.  The crab fishery wanted us to be north of North Head and we wanted to be south of it.  The compromise was to make a big site on both sides.  Peacock Spit is very energetic and that is where the waves are breaking most wildly.  Operationally, if we want to have a site for dredge material placement it needs to be reliable.

- Terry Geroux said material shifts and things start to get dangerous.

- Rod said the operating area of Peacock Spit is the most problematic for the dredge.  He said that doesn’t mean we should consider any other area. Rod suggested moving forward with the North Head Site or some semblance of it because it is dispersive.  He said maybe we should pursue placing material directly where the models show would be most beneficial, which is George’s proposed area.

- Terry said at a quick glance he doesn’t know if we could make it work with the dredge.  Jarod said we would have to talk to the captains.

- Jarod noted that Rod had mentioned the extension of the jetty by 200 feet and asked if that is going to change any of these models.  He asked if we need to do another couple iterations of disposal with the 200 foot longer jetty.  He said we are not sure.

- Rod said within George’s suggested box, the depths are 40-45 feet which can be dicey with the swells coming from Peacock Spit.

- Jim said we need to look at the historical data from this group as to why we moved away from the area George suggested.  But we have a designated the North Head Site approved for a pilot study. He asked if we can we still use it the next two years under the current Cert and expand the volume to inform our decision.

- Rod said we could possibly slide the location south.

- Penney said moving the box is an issue.  She said we can change the box, but we would have to go through the process.

- Jarod suggested we could move it slightly to include some of the area and do disposal on good days.

- James McMillan said we can see that the southeast corner of the box is the most energetic.  He said we placed in the top of the lower third previously.  He said perhaps we could go to the southern boundary and still be within the Cert.  He said this could give us information outside the box while staying within it.

- Penny said she’s not sure we can stretch the Cert into a phase III, but we could do an amendment to say it will give us more information on how to use the site.  We could consider Rod’s proposal of three sections.

- Brian said that we’ll need to convene the technical team to advise on how we can get outside that box or not.  We need information where the shipwreck is in that area.  Jarod said it may be very well littered with shipwrecks so we should look at that first to see if it’s even possible.

- Penny said we could either amend or ask for phase III as long as we stay in the box but do another third pilot study on another area within the box with the group’s support,

- Brian said we would want to make sure we are learning something new from this next pilot study.

- Penney said if you want to call it phase III and you will need to give us a monitoring report and what you are going to learn from it.

- James McMillan said we could learn something by going to the very southern end of site.

- Rod suggested a two-foot accumulation with thin layer, long feature and much wider, so we can monitor it.

- Brian said this is still better than using the Deepwater Site.  He said there is still positive beneficial use.  A good next step would be to focus on a longer-wider feature in the southern border.  Brian said he would like the Technical Team to delve into this more to look for any constraints.

Benson Beach Strategy Going Forward

- Brian said we keep coming back to on-shore disposal at Benson Beach because we know the best way to get sand into the system is to put it directly on the beach.  He said we’re hearing there may be some options around this.  In 2010, we put 390k cubic yards on Benson Beach and this only happened because we had state and federal funds.  It was a one-time project to learn about onshore placement on Benson Beach.  We were successful because this group was behind it.  It’s unlikely that there are options for ongoing state funds, but we could try again.  He said there are a lot of steps to get something from us to the Governor’s office.  He said it would be good to explore other funding options as well. Her asked if there is a case to be made for the South Jetty Site and Benson Beach.

- Jarod said jetty repairs were going on for the last four years, so we couldn’t seek Benson Beach funding during that time.  He said this onshore placement project goes back into the budget request every year and we have to start fighting for it at the higher levels.  He said we need to get people excited about it.  Jarod said it gets denied because it’s not part of our navigation needs, but it’s bad enough now that sand definitely needs to be placed on the beach.

- Brian asked if there is an argument that it needs to be there to protect the jetty.  Jarod said not necessarily but the loss of economic resources for State Parks, safety to the public and loss of infrastructure are all valid issues.

- Dale asked if the Corps has done a recent analysis of how much they are saving by using the North Head, South Jetty and Shallow Water sites rather than the Deep Water Site.  He said these actions do result in a net savings to the Corps.  He asked if that could be applied every two or three years to onshore placement at Benson Beach.  Jarod said it absolutely is a savings to the Corps but we can’t save up savings.  The Corps looks at it as savings to the entire Corps budget.

- Jarod said all the other critical needs of impacts seem like enough justification to try to push this through.  We need to make sure that it is on everyone’s radar and everyone in this group using the same messaging.

- Brian asked about tangible next steps for Benson Beach.  He said we could start with the Corps, but maybe after some work to identify what that shared request so that we can start to advocate in appropriate places.

- Jarod said maybe a letter from LCSG outlining concern about Benson Beach to get the leadership engaged.

- Karla Ellis said if there was a standalone justification it would not fly but having a group of stakeholders on the same message would really help.

- Lauren Bavernschmidt said she is newer to this group.  She asked how long the sand stayed in place after the onshore placement at Benson Beach.  Jarod said a lot of material went away within two months or so because of a big storm but stayed in the system for one to two years.  Jarod said it is good to think about ways to help keep it on the beach.  Rod said with the 2010 design they built a bulge.  He said if we wanted to keep it in place, we could build a ribbon.  He said ribbons stay, while bulges provoke wave events and take away more than the ribbon placement. Bobbak said that project was designed to have a dynamic revetment.

- Jarod said quantifying the potentially losses and impacts that the beach is having and coming up with a unified message is critical within the next month of two.  He cited the efforts of the City of Lake River to be vocal about a project that isn’t as important as this, but it will not come off the Corps radar because they are a vocal group.

- Bobbak said if the Corps budget cycle starts about a year in advance of session, when do we need to submit a request.  Kate said their budget 2022 has already been submitted, but they could probably still submit a work package for 2022.  She said they would need to come up with a plan asap.  Carla said they could let the program folks know that something may be coming in and to hold off on submitting the final work package yet.

- Bobbak asked what the logistics involve.  He asked if Ecology needs to develop an application or does the Corps do this with our support.  Jarod said it would start with the Corps and we need to come up with messaging as a group.  He said Kate will take the lead on this and will look at the justification from the previous effort as well.

WRAP UP AND NEXT STEPS

- Brian said WDOE and several other partners have funded the facilitation role, but we are not going to do so through state cost sharing again.  He said the Corps is going to pick this up and put out bid for proposal for new facilitator.  He said this is Jim’s final meeting due to retirement.  Brian said they willing to help with that process, and it is important to get a very strong facilitator.  We have Jim through May then we are rudderless.  He said if we don’t have somebody we will lose momentum.

- Brian said Jim is going to be working on updating RSMP.  He said a lot of the existing content and information is relevant but needs to be updated. Jim said we will be querying members about what elements you would like updated and how, then we will draft something and circulate.  He said it will then come back to the LCSG for approval.

- Brian said all the things we said we were going to do in the plan, we have been doing. He recommended going back and reading the plan.

- Brian said we might convene the technical team at least in a conference call.

- Brian guesses we will probably meet again in June and December.

- Big thanks again to Jim.  We would not be here today without his support.  He has been vital to this process.

- Jim thanked the group for all they have done over the last 12 years he has been involved.

- Brian thanked Jarod, also in his last meeting, as he has been instrumental to moving this group forward.

Meeting adjourned at 3:11 pm.